**  Simple RC Car with Arduino Nano + L298N + Joystick  **

** Overview :

This Arduino project controls a 4-motor RC car (2 motors per side in parallel) using one joystick:

- Forward/Backward movement → vertical axis of joystick (JOY_Y / A1)

- Left/Right steering → horizontal axis of joystick (JOY_X / A2)

The L298N motor driver handles the motor control with PWM for speed regulation.

** Components:

- Arduino Nano

- L298N Motor Driver Module

- 4 x N20 DC Motors (2 left, 2 right)

- Joystick Module

- Battery Pack (suitable voltage for motors and Arduino)

- Jumper Wires

- Breadboard

** Circuit Connections:

* L298N Motor Driver:

- ENA (PWM for left motors) -> Arduino D5

- IN1 (Left motor direction 1) -> Arduino D2

- IN2 (Left motor direction 2) -> Arduino D3

- IN3 (Right motor direction 1) -> Arduino D4

- IN4 (Right motor direction 2) -> Arduino D7

- ENB (PWM for right motors) -> Arduino D6

- OUT1 -> Left motor + (connect both left motors in parallel)

- OUT2 -> Left motor - (connect both left motors in parallel)

- OUT3 -> Right motor + (connect both right motors in parallel)

- OUT4 -> Right motor - (connect both right motors in parallel)

- VIN -> Positive terminal of battery pack

- GND -> Negative terminal of battery pack and common ground with Arduino and Joystick

* Joystick Module:

- VCC -> Arduino 5V

- GND -> Arduino GND (common ground)

- VRx -> Arduino A2 (for steering left/right)

- VRy -> Arduino A1 (for throttle forward/backward)

* Arduino Nano Power:

- Power Arduino via USB or 5V regulator

- Ensure common GND with L298N and Joystick

** Controls :

* Forward : Turn Left

* Backward : Turn Right

* Right : Go Backward

* Left : Go Fordward