** CODE =

// ----- PIN DEFINITIONS -----

// L298N - LEFT MOTOR

#define ENA 5

#define IN1 2

#define IN2 3

// L298N - RIGHT MOTOR

#define ENB 6

#define IN3 4

#define IN4 7

// Joystick

#define JOY_Y A1   // Forward/Backward

#define JOY_X A2   // Left/Right

int deadzone = 80;

// ----- SETUP -----

void setup() {

pinMode(ENA, OUTPUT);

pinMode(ENB, OUTPUT);

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

Serial.begin(9600);

}

// ----- MOTOR FUNCTIONS -----

void leftMotor(int speed) {

if (speed > 0) {

digitalWrite(IN1, LOW);

digitalWrite(IN2, HIGH);

}

else if (speed < 0) {

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

}

else {

digitalWrite(IN1, LOW);

digitalWrite(IN2, LOW);

}

analogWrite(ENA, abs(speed));

}

void rightMotor(int speed) {

if (speed > 0) {

digitalWrite(IN3, HIGH);

digitalWrite(IN4, LOW);

}

else if (speed < 0) {

digitalWrite(IN3, LOW);

digitalWrite(IN4, HIGH);

}

else {

digitalWrite(IN3, LOW);

digitalWrite(IN4, LOW);

}

analogWrite(ENB, abs(speed));

}

// ----- MAIN LOOP -----

void loop() {

int rawY = analogRead(JOY_Y);  // Forward/Backward

int rawX = analogRead(JOY_X);  // Left/Right

// Convert joystick to motor speed

int throttle = map(rawY, 0, 1023, 255, -255);  // Push UP = forward

int steer    = map(rawX, 0, 1023, -255, 255);  // Left/Right steering

// Apply deadzone

if (abs(throttle) < deadzone) throttle = 0;

if (abs(steer) < deadzone) steer = 0;

// Motor mixing

int leftSpeed  = throttle + steer;

int rightSpeed = throttle - steer;

// Limit speed range

leftSpeed  = constrain(leftSpeed, -255, 255);

rightSpeed = constrain(rightSpeed, -255, 255);

// Run motors

leftMotor(leftSpeed);

rightMotor(rightSpeed);

delay(10);

}